APPENDIX B.5

RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 1 of 5)

DO NOT WRITE IN THIS BLOCK

Project: 		____PASE_________		Installation Period:	__6/11/07-7/1/07___

Aircraft: 		____C-130________		Beginning Date: 	__8/4/07__________

Instrument Number:	_________________		Ending Date: 		__9/8/07__________

1. 	Instrument: 		__Fast-response Ozone___________________________________

2.	Function: 		__high rate ozone for fluxes and mixing ratios________________

3.	Principal Investigator:	__Teresa Campos_______________________________________

Address: 		__RAF_______________________________________________

Telephone: 		__x005F_x1048_______________________________________________

4. 	Instrument Operator(s): _Teresa Campos, Ilana Pollack, Cliff Heizer

5.	Is this instrument commercially produced? 	_No________________________________

6.	If so, please list name and address of manufacturer:

________________________________________________________________________

7. 	Please list serial number of the instrument:

________________________________________________________________________

Please attach a copy of the manufacturer’s instruction manual for the device.  If this is not possible, attach a copy of those pages of the instruction manual which describe the principles of operation, hazard warnings, safety features, and safety rules.

8. 	If the instrument is not commercially produced, please provide information requested below:

Designed by: 	I Pollack, T Campos, A Weinheimer (after Ridley, et al)

Organization: 	_NCAR______________________________________________

Address: 		_RAF________________________________________________

Telephone: 		_x005F_x1048_______________________________________________

Built by: 	            _Pollack, Campos, Weinheimer and CARI Group___________

Organization:         __NCAR______________________________________________

Address: 	            __RAF_______________________________________________

###### APPENDIX B.5

# RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 2 of 5)

9.	Describe principles of operation, hazard warnings, safety features:

Quantifies ozone via chemiluminescent reaction with highly concentrated nitric oxide

(NO).

Hazards: 1) NO is a very toxic gas; although not flammable itself, it is an oxidizer and

accelerates an existing fire.  MSDS is included.  Precautions have been taken minimize

the risk of crew exposure to unsafe levels of NO, as described in #25.  2) Instrument uses

dry ice as a coolant prior to and during flights.  About 5 lbs are used for a 9 hr flight.

10. 	If the instrument is commercially produced, has it been modified? 	____No___________

11. 	If modified, describe the modification.

________________________________________________________________________

All investigators please answer the following:

12. 	Does the instrument contain, use, or produce:

Radioactive materials		_No_____	Compressed gases		__Yes____

Other ionizing radiation	_No_____	Non-ionizing radiation	__No____

Flammable liquids		_No_____	Laser				__No____

Radar				_No_____	Flammable gases		__No____

Explosive materials		_No_____	Toxic materials		__Yes____

13. 	If any of the categories were checked, specify the material below (for example, amount, energy levels, physical form, etc.).

__We plan to carry 500 psi of 99.995% NO in a compressed gas cylinder having 0.44 L internal volume.__________________________________________________________

14. 	Please list all other chemicals you will use on board this aircraft in your experiment.

________________________________________________________________________

15.	If your experiment consumes or discharges materials, will you need to carry additional materials on board?   No.

###### APPENDIX B.5

# RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 3 of 5)

16. 	 What and how much extra materials will you need to carry?

_None.__________________________________________________________________________________________________________________________________________________________________________________________________________________

17. 	What kind of container will you need to carry these materials?

________________________________________________________________________

18. 	If the device utilizes a laser, please classify the laser according to ANSI Z 136.1-1973 (circle one).

Class:	I     II     III     IV

- If your laser will be operating at a wavelength that is not eye safe,  what procedures will

be established to minimize the danger to yourself and other project participants?

______please attached a separate document covering this question__________________

20.	If you are using compressed gas cylinders, what is the maximum pressure expected for each cylinder type?

__500 psia pure NO_______________________________________________________

21.	Will you be re-filling any compressed gas cylinders yourself, either at JeffCO or during the field deployment?

__No___________________________________________________________________

22. 	Are there any other hazards associated with the instrument itself, the required ground support equipment or the experiment which have not so far been covered in this questionnaire?

_The 8 hr TLV exposure limit for NO is 25 ppmv.  If a leak occurs in NO plumbing line, crew can be at risk.

23. 	How would you describe the probability of an accident resulting from the presence and use of your instrument on board the NCAR aircraft?

_________________________________________________________________________Low probability, if precautions discussed in #25, below are implemented.

###### APPENDIX B.5

RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 4 of 5)

24.	How would you describe the severity of such an accident?

_Without #25, such an accident could be quite severe.  With precautions, the risk of accidental exposure is quite low.

25. 	What precautions will you take to decrease the probability and the severity of an accident?  If any documented safety procedures from your home facility or university are available, please attach a copy of said materials to this form.

NO handling precautions and procedures:

1) A lecture bottle of compressed NO gas, a pressure regulator, a low pressure shutoff valve and several safety measures are housed inside a containment vessel to prevent leakage of NO into the cabin in the event of cylinder valve or regulator failure.  (For additional safety information, emergency safety procedures, and vessel operations, please see the attached certification document for the containment vessel.  Document # 677_05_O3C_200_FM_rev7.doc)  Before installation on the aircraft, the containment vessel leak rate is measured to ensure that it can hold 60 psi of positive pressure for 1 hour without detectable pressure change.  When in use on the aircraft during flight, the containment vessel is vented overboard to allow over pressurized contents to release to air outside the aircraft cabin.  A flow restrictor has been installed inside the vessel to guarantee that in the event of line rupture, crew exposure cannot exceed 8-hr safety limit.

2) NO-containing components within the cabin are leak tested to assure adequate

isolation from cabin environment.  No in-flight modifications to plumbing will occur.

3) NO leak detectors with audio and visual alarms are mounted on the equipment rack near the instrument to provide early warning for crew.

4) NO will not be allowed to flow during pre-flight ground operations so as to minimize risk of exposure to ground crew.  The procedure for turning on NO flow requires that pre-flight engine start be first initiated.  Twenty minutes before flight’s end, NO flow is stopped and given time to be flushed from the system prior to landing and engine shutdown.  NO gas will only be allowed to flow while the aircraft is on the ground during instrument calibration days.  During these days, a pumped system with a long hose will be used to lead exhaust to a safe distance away from the aircraft.

###### APPENDIX B.5

RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 5 of 5)

Cryogenic CO2 handling procedures:

- Operators will use gloves and eye protection to prevent cryogenic burns while

handling dry ice.

_________________		________________________________________________

Date				  Signature of principal investigator or operator

________________________________________________

Printed name of principal investigator or operator

________________________________________________

Reviewed by

________________________________________________

Date