3/07/12

Aircraft Research Instrumentation

C-130 - Basic RAF IDEAS-4 Configuration

Stith et al

RAF-Supplied Instrumentation:

I.  Airborne Data System.

A. Acquisition.  C-130 NIDAS. NCAR Distributed Sampling

Modules (7 units).

B. Operator Station. Intel L440GX+ Motherboard with

2 Pentium III-500 CPU’s, 512 MB Memory, Redhat

Linux 6.2 OS.  9GB Disk Storage with RAID-1.

General Digital 17 inch 1280x1024 TFT Flat Panel

Color Monitor.  IBM-DRHS36D 36GB Removable Disk Drives

In Kingston Carriers (2 units). UPS.

II. Aircraft Position, Velocity and Attitude

A. Honeywell Model HG1095-AC03 Laseref SM Inertial

Reference System . (Research).

B. Garmin GPS16 Global Positioning System. (GPS).

C. NovAtel Model OEM 4 L1/L2 Differential GPS High Accuracy

Positioning System (DGPS)

III. Static Pressures.

A. Paroscientific Model 1000 Digital Pressure Transducer -

Fuselage Port (PSFD, PSFDC).

B. Paroscientific Model 1000 Digital Pressure Transducer -

Fuselage Port (PSFRD, PSFC).

C. Rosemount Model 1201F1 Pressure Transducer –Cabin (PCAB).

D. Rosemount Model 1201F1 Pressure Transducer –CN Counter (PCN).

IV. Dynamic Pressures.

A. Rosemount Model 1221F1VL - Radome (QCR).

B. Rosemount Model 1221F1VL – Left Fuselage Pitot (QCF).

C. Rosemount Model 1221F1VL – Right Fuselage Pitot (QCFR).

V.  Temperatures.

A. Rosemount Type 102 Non-deiced Sensor -- Rosemount Model

510BF Amplifier - Port Radome Mount (TTRL).

B. HARCO Deiced Sensor -- Rosemount Model

510BF Amplifier - Left Fuselage (TTHR1, TTHR2).

D. OPHIR3 Radiometric Temperature Sensor – Right Wing Pod (OAT).

E. RAF Experimental In Cloud Temperature Sensor

VI. Flow Angle Sensors, Radome.

A. Attack - Rosemount Model 1221F1VL Differential Pressure

Transducer (ADIFR).

B. Sideslip - Rosemount Model 1221F1VL Differential

Pressure Transducer (BDIFR).

C-130 RAF IDEAS-4

Page 2

VII. Dew Point and Humidity.

A. Buck Research, Model 1011C Dew Point Hygrometer –

Starboard Radome Mount (DPT).

B. Buck Research, Model 1011C Dew Point Hygrometer - 	    	   	   Starboard Radome Mount (DPB).

C. RAF Fast Response UV Hygrometer – L3 aperture (XUVI)

D. RAF Cryogenic Hygrometer – (MR_CR2)

VIII.   Radiometric Sensors.

A. Remote Surface Temperature.  Heiman Model KT19.85

Radiometer - 2 units: Down looking (RSTB,RSTB1).

B. Remote Sky Temperature.  Heiman Model KT19.85 Radiometer.

Up looking (RSTT).

C. Infrared Irradiance. 2 units top and bottom (IRT, IRB).

IX. Wing Stores

A. RAF – DMT UHSAS Aerosol Probe.

Size Distribution, 0.06 - 1 um. RWO.

B. RAF - PMS 2D-C Optical array Probe

Size Distribution, 25 to 1600 um. LPC.

C. RAF – Tip Modified PMS 2D-C Optical array Probe

Size Distribution, 25 to 1600 um. LPI.

D. RAF - Particle Measuring Systems Model SPP-300

Size Distribution, 0.3 - 20 um. LWI.

E. DMT – CDP Cloud Droplet Probe

Size Distribution, 3 - 45 um. LWO.

F. RAF – 1D Laser Air Motion Sensor (LAMS). RPO.

G. HAIS Small Ice Detector (SID-2). RWI.

H. SPEC Cloud Particle Imager (3V-CPI). RPI.

I. Holographic Cloud Particle Imager (HOLODEC-2). LPO.

X.  Basic Cloud Physics.

A. PMS Liquid Water Sensor (King Probe) -

Left Outboard Wing Access Plate (PLWC).

B. TSI Model 3760 CN Counter - Belly Inlet (CONCN).

Size Distribution, 0.01 to 3 um.

C. Rosemount Icing Rate Indicator

Right Outboard Wing Access Plate (RICE).

D. Modified TSI Water CN Counter

E. Modified TSI Water CN Counter

F. PCASP – Optical Aerosol Particle Analyzer.

XI. Photography.

- Forward Looking Digital.

- Down Looking Digital.

C-130 RAF IDEAS-4

Page 3

XIII. RAF Chemistry (T.Campos).

- EOL Fast Ozone (FO3_ACD).

- EOL Picarro Carbon Dioxide (CO2_PIC).

- EOL Picarro Methane (CH4_PIC).

- EOL Carbon Monoxide (CORAW_AL).

- HAIS POA Ozone Analyzer.

XIV.	 User Supplied Equipment

- CSU CFDC Ice Nuclei Counter.

- CU Picarro Water Isotope.