APPENDIX B.7

RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 1 of 4)

DO NOT WRITE IN THIS BLOCK

Project: 		___NOMADSS___		Installation Period:      8 Apr – 22 May 2013

Aircraft: 		____C-130_______		Beginning Date: 	__1 Jun 2013______

Instrument Number:	_________________		Ending Date: 		__15 Jul 2013_____

1. 	Instrument: 		____SMPS_(Scanning mobility particle sizer)____________

2.	Function: 		_Aerosol measurements:  Particle size distributions___

3.	Principal Investigator:	_____James N. Smith_______________________________

Address: 		_____NCAR/ACD; PO Box 3000, Boulder, CO  80307____

Telephone: 		_____(303) 497-1468_______________________________

4. 	Instrument Operator(s):   John Ortega, Jim Smith

5.	Is this instrument commercially produced? 	As a whole, no.  Portions of the instrument are commercially produced, while others were fabricated and assembled at NCAR.

6.	If so, please list name and address of manufacturer:

DMA and CPC made by TSI, computer by Diamond Systems, Power supplies by Vicor.

7. 	Please list serial number of the instrument:

___  NCAR PPE number 000011797________

Please attach a copy of the manufacturer’s instruction manual for the device.  If this is not possible, attach a copy of those pages of the instruction manual which describe the principles of operation, hazard warnings, safety features, and safety rules.

8. 	If the instrument is not commercially produced, please provide information requested below:

Designed by:

Organization: 	__James N. Smith, NCAR/ACD  _____________________

Address: 		__NCAR/ACD; PO Box 3000, Boulder, CO  80307______

Telephone: 		___(303) 497-1468________________________________

Built by: 	            _same as above__________

Organization:         _same as above __________

Address: 	            _same as above___________

###### APPENDIX B.7

# RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 2 of 4)

9.	Describe principles of operation, hazard warnings, safety features:  Particle-containing air is brought into the instrument and separated into a sample stream and sheath air stream.  The sample stream goes through an ionizer to charge particles that are then classified according to mobility diameter (~10 nm to 100 nm) using a high voltage source within a differential mobility analyzer (DMA).  Upon exiting the DMA, the size-selected particles are counted using a condensation particle counter (CPC, commercially produced by TSI), and the counts (and other relevant sample information) is stored on a PC104 computer that sits on the same rack.  All electrical components that might fail have been properly protected by circuit breakers.

10. 	If the instrument is commercially produced, has it been modified? 	___________

11. 	If modified, describe the modification.

All investigators please answer the following:

12. 	Does the instrument contain, use, or produce:

Radioactive materials		__x___	Compressed gases		______

Other ionizing radiation	______	Non-ionizing radiation	______

Flammable liquids		__x___	Laser				__x___

Radar				______	Flammable gases		______

Explosive materials		______	Toxic materials		______

- If any of the categories were checked, specify the material below (for example, amount, energy levels, physical form, etc.).

- CPC working fluid = N-butanol; liquid remains inside CPC reservoir; ~50 ml used per 6 hour flight.  Vapors are pumped out of the cabin via the exhaust.

- Sealed Po210 source ( radiation at 10 mCi); only sample stream is exposed to this source.

- CPC (TSI 3010) has enclosed Class 1 laser for particle counting.  Class 1 lasers are considered safe under all normal operating conditions, and it is completely encased within the instrument housing.

14. 	Please list all other chemicals you will use on board this aircraft in your experiment.

n/a

15.	If your experiment consumes or discharges materials, will you need to carry additional materials on board?    n/a

###### APPENDIX B.7

# RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 3 of 4)

16. 	 What and how much extra materials will you need to carry?   n/a

17. 	What kind of container will you need to carry these materials?

___________________n/a_________________________________________________

18. 	If the device utilizes a laser, please classify the laser according to ANSI Z 136.1-1973 (circle one).

Class: I

- If your laser will be operating at a wavelength that is not eye safe,  what procedures will

be established to minimize the danger to yourself and other project participants?

_Laser for particle counting is completely enclosed within CPC.  No light can get out._

20.	If you are using compressed gas cylinders, what is the maximum pressure expected for each cylinder type?

_______________n/a______________________________

21.	Will you be re-filling any compressed gas cylinders yourself, either at JeffCO or during the field deployment?

___________________________NO______________________________________

22. 	Are there any other hazards associated with the instrument itself, the required ground support equipment or the experiment which have not so far been covered in this questionnaire?   NO

23. 	How would you describe the probability of an accident resulting from the presence and use of your instrument on board the NCAR aircraft?  Low probability; this type of instrument has been used for decades in the laboratory, field studies and on aircraft.  We know of known accidents related to the instrument’s standard operation.

###### APPENDIX B.7

RAF PROJECT SAFETY COMMITTEE

HAZARDOUS MATERIALS AND DEVICES

(Page 4 of 4)

24.	How would you describe the severity of such an accident?  Not severe or n/a.

25. 	What precautions will you take to decrease the probability and the severity of an accident?  If any documented safety procedures from your home facility or university are available, please attach a copy of said materials to this form.  All materials and components have been approved to fly on the GV.  Required documentation has been reviewed, submitted and approved.

_________________		________________________________________________

Date				  Signature of principal investigator or operator

________________________________________________

Printed name of principal investigator or operator

________________________________________________

Reviewed by

________________________________________________

Date