DEEPWAVE Project Manager Report

Aircraft payload

Cabin:

VCSEL

Rayleigh Llidar

Na Lidar

LAMS (exper.)

CR-2 (exper.)

MTP

CN counter

MTM (Utah)

AVAPS

LWO:

GUST pod

LAMS

RWO:

UHSAS

MTP

RICE

General Data notes

DEEPWAVE is a night time project. There is no forward looking video or digital imagery produced for this project due to low visible light levels. There are digital images available for RF15, TF01, FF02, FF04, and FF05; these can be made available by request.

This Applanix inertial reference system was flown in DEEPWAVE primarily as an experiment to verify its performance and viability as a potential compliment to the other GPS and Inertial systems on the GV as it has accuracy specifications that exceed the others. The Applanix performed well on the test flights but in the field it only performed satisfactorily on 7 out of 26 research flights. These were flights RF07, 8, 13, 15, 23, 25, 26. The problems have not been fully diagnosed but appear to be related to lack of reliable GPS on some flights and an inconsistent start up and initialization procedure on others. It should also be noted that there was a lack of expertise on this instrument in the field and it was the first time it had been installed and flown on the GV in this configuration. The data from the Applanix were recorded on a computer separate from the primary aircraft data system (ADS) server.

The RICE supercooled liquid water (SLW) indicator was affected by a noisy power or ground, traced to the interference with a co-located UHSAS instrument. The signal from RICE may become negative on occasion. The impact of this on data quality is low as there were very few encounters of the SLW during the project; RICE should be used only as a qualitative indicator of onset of icing.

During DEEPWAVE the chilled mirror hygrometers operated in regimes exceeding their design limits. After descents they frequently oscillated trying to stabilize the mirror temperature in presence of excessive condensation. After RFxx additional insulation was added to the dew pointers and their performance improved. For all flights, at high altitudes the frost point is lower than the sensors’ cooling capability and the instruments reach their limit at -65 to -70C.

For the first 4 flights there were problems switching between the two OmniSTAR satellites with the result that OmniSTAR is fully active for 50-70% of the flight time for these flights. After configuration changes to the GPS receiver the fully-corrected coverage increases to nearly 100% for the remainder of the flights. The flights where OmniSTAR dropped out have been post-processed using the ground reference stations.

The UHSAS suffered from insufficient flow, intermittent internal cabling/connection problems, and occasional spikes in the histogram on many flights during this project. The cabling/connection issues affected the reported housekeeping values but not actual values, and nearly all of these incidents were detected and corrected in processing.

Flow problems, on the other hand, led to data losses for nearly half the flights.  Through the project, the UHSAS increasingly could not maintain adequate sample and sheath flows at low ambient pressure.  No viable data were obtained from RF09 through RF19 as a result.  After RF19 the UHSAS pump was replaced, restoring the nominal instrument performance for the remaining flights. Size distribution histogram spikes are an infrequent occurrence, and generally persist for only 1 second, but occasionally up to 4 seconds.  These events have been noted on individual flights below (where obvious) and blanked out.

The counting efficiency of TSI 3760A butanol CN counters declines rapidly with pressure below about 200 mb, where much of the DEEPWAVE flying took place.  A pressure-dependent correction based on published measurements has been applied.  The true correction is size-dependent up to about 50 nm or so, but since the CN counter lacks size information we are limited to a single value applied uniformly to all sizes.  The applied correction factor is therefore conservative, and may lead to slight underestimation of the very low concentrations often seen at high altitude. The instrument used the PDUMPPL sensor, plumbed to its intake, to provide CN pressure CNPS ('Cn Pressure Static') for this project only.

The gust pod wind measurement system uses an internally mounted inertial system, which is compact but inferior in performance to the aircraft INS. This CMIGITS INS sometimes shows drift in the heading measurement, which can render wind calculations incorrect. In all cases CTHDG_GP should be compared with THDG and while a stable offset is normal, rapid excursions should be a flag to disregard wind data from the gust pod (*_GP). All _GP wind data when ROLL exceeds 5 deg should also be disregarded and have been removed from the dataset. Further, CROLL_GP showed several instances of slow drift that is difficult to identify, causing a slow departure of WI_GP from WI and WIC. These periods are mentioned in flight specific notes below.

TAS_GP, the air speed measurement from the gust pod, was computed for the project utilizing solely the attack, static pressure and dynamic pressure from the gust pod, and ambient air temperature. The parameterization of TAS_GP for PS_GP is limited to altitude above 5,000 m, true airspeed over 130 m/s and aircraft roll angle less than 5 deg. All TAS_GP data outside of this envelope have been removed from the data files.

AKC_GP is a surrogate angle of attack calculation that utilizes the attack angle computed from the gust pod to approximate the attack angle of the radome gust probe using a simplified equation

AKC_GP = 3.73 + 1.1 * AK_GP

This approximation is derived from flights preceding the servicing of the radome after RF06, is representative of the flights before RF06 and has been used to compute the winds from the radome for RF06 and RF07. These are the flights during which ADIFR, the vertical differential pressure measurement on the radome, was compromised by frozen water droplets in the tubing and resulted in inability to calculate WI and all other wind vectors from the radome probe. Therefore the WI and WIC for RF06 and RF07 are computed using the gust pod angle of attack, yet the lateral wind vectors and wind speed are computed from the radome gust pod.

Flight Specific Data notes

RF01

1/11/2014

The backup GPS (Garmin) did not function from 08:00 to the end of the flight. Omnistar service dropped out from 06:18 to 07:08; 07:47 to 08:10; 08:18 to 08:50; 09:07 to 10:16; 11:41 to end of flight. From the comparison with the avionics GPS (ALT_A) it appears that only the period from 09:07 to 10:16 sustained loss of accuracy in the GPS measurement. From the comparison with the avionics GPS it does not appear that other dropout periods sustained a loss of accuracy (there are no step changes observed in GGALT near the time of onset of DGPS augmentation loss, which are indicative of the unresolved L1-L2 ambiguity).

DPL became unstable and did not produce usable data after 09:11.

UHSAS flows dropped at high altitude.  Pump pressure relief valve adjusted prior to this flight, 2 turns CW (supposed to raise opening pressure).  Flows declined at slightly lower pressure than during test flights, suggesting that relief valve adjustment was in wrong direction.  Some data loss mid flight due to bad flow and a power cycle.

CVSPD_GP exhibited an unexplained drift from 08:22:59 to 08:53:24, causing bad _GP wind data during that period.

RF02

1/13-1/14/2014

The Omnistar service dropped out from 07:35 to 08:00; 10:13 to 11:25; 11:51 to 13:20. From the comparison with the avionics GPS it does not appear that there was a loss of accuracy during the dropout periods (there are no step changes observed in GGALT near the time of onset of DGPS augmentation loss, which are indicative of the unresolved L1-L2 ambiguity).

UHSAS pressure relief valve on pump readjusted 4 turns CCW, to undo previous change and add two turns in reverse direction, prior to this flight. A few spikes in CONCU due to an internal serial bug; these are blanked out. Flows sagged at 40 kft, but otherwise data look OK. There was one episode, < 5 minutes, of negative flow (or more likely zero, based on zero particle counts) at 13:58:59-14:03:13. Noted rise in CONCU from ~80/cc to ~500/cc at about 10:30-11:20 and again at 12:10-13:05, corresponding to transects over and east of Tasmania, respectively.  Size distributions look good.

_GP winds show two periods of significant deviation from the radome winds. These are likely attributable to a CPITCH_GP drift that is not consistent and can’t be readily identified. These periods are from 08:35 to 10:11 and from 13:28 to 15:16.

RF03

1/17/2014

The Omnistar service dropped out from 08:03 to 10:14. From the comparison with the avionics GPS it does not appear that there was a loss of accuracy during the dropout periods (there are no step changes observed in GGALT near the time of onset of DGPS augmentation loss, which are indicative of the unresolved L1-L2 ambiguity).

UHSAS Sheath flow sagged for much of the flight, but sample flow was maintained.  Some noise at times in the flows ~10%; also noise in other housekeeping parameters.  A few spikes once again, but data are otherwise good.

DPL stopped outputting valid data after 10:40:15.

All _GP wind data are bad for this flight due to the bad pitch and roll data produced by the gust pod inertial system, and have been removed from the dataset.

RF04

1/19/2014

Left dewpointer was inoperable for the entire flight.

The Omnistar service dropped out from 06:49 to 10:00; 11:05 to 11:35; 12:05 to 12:22; 14:24 to end of flight. From the comparison with the avionics GPS it does not appear that there was a loss of accuracy during the dropout periods (there are no step changes observed in GGALT near the time of onset of DGPS augmentation loss, which are indicative of the unresolved L1-L2 ambiguity).

Gust pod inertial system did not initialize properly on take off, which resulted in a scrambled heading measurement and caused the loss of all _GP wind data for this flight.

RF05

1/22/2014

The Omnistar service dropped out from 10:22 to 10:41 and from 14:38 to the end of flight. From the comparison with the avionics GPS it does not appear that there was a loss of accuracy during the dropout periods (there are no step changes observed in GGALT near the time of onset of DGPS augmentation loss, which are indicative of the unresolved L1-L2 ambiguity).

Left dewpointer was noisy for extended periods at high altitude where the frost point temperature exceeded the instrument operating range. Refer to DP_VXL for frost point measurements.

RF06

1/24/2014

The Omnistar service dropped out from 08:56 to 09:30 and from 10:20 to 11:00.

UHSAS generally ran well, but suffered decreased flows late in the flight, from about 12:37:00 - 13:32:40.  Sample flow went to zero and particle counts were at zero during this period.  Sample flow recovered at about 13:28:30, but particle counts did not resume until 13:32:42.  Normal operation then continued for the remainder of the flight.

Radome gust pod ingested water into the ADIFR ports on take off and the attack measurement was severely biased the entire flight. The gust pod attack angle measurement was unaffected and is used as a reference for PSXC calculation for this flight. AKC_GP was used as a reference attack angle to calculate W, U and V vectors for this flight.

_GP winds show one period of significant deviation from the radome winds. It is likely attributable to a CPITCH_GP drift that is not consistent and can’t be readily identified. This period is from 07:24 to 08:37.

RF07

1/24/2014

CN began to run out of butanol starting around 07:00 (cannot pinpoint the time), when CNCNTS began an abnormal slow decline in level flight, dropping below ~10 Hz by about 12:30.  Apparently normal operation up to 07:00, but even those data are suspect.  All subsequent CN data for this flight are discarded.

VCSEL data lost from 05:59:57 to 06:11:00.

Radome gust pod water in the ADIFR ports was only partly corrected and in flight the ports froze, causing the attack measurement to be severely biased the entire flight. The gust pod attack angle measurement was unaffected and is used as a reference for PSXC calculation for this flight. AKC_GP was used as a reference attack angle to calculate W, U and V vectors for this flight.

DPR did not produce usable data for the entire flight.

RF08

1/24/2014

DSM 310 failure at about 09:38.  No CN data for remainder of flight. UHSAS were lost from that point on as well. UHSAS sample flow dropped out within ~90 minutes of take-off.  Power cycling failed to correct the issue.

GPS data gap exists from 10:07 to 10:10.

RF09

1/24/2014

No UHSAS data for this flight.  Sample and sheath flows were low on power up, only about 60% of normal, and never recovered.  Sample flow dropped quickly to near zero on climb out.

The Omnistar service dropped out from 10:35 to 10:40.

_GP winds show one period of deviation from the radome winds. It is likely attributable to a CPITCH_GP drift that is not consistent and can’t be readily identified. This period is from 08:22 to 08:52.

RF10

2/8/2014

No valid UHSAS data at altitude for this flight.  Again sample and sheath flows were low on start up, falling further during climb out, with sample flow dropping to near zero.  In addition, flow readings were pegged at their maximum values for much of the flight (while actual flows remained extremely low).

The Omnistar service dropped out from 08:02 to 08:04

WI and WIC show a positive offset at the lower altitude leg from 10:15 to 10:55 due to an imperfect altitude dependent parameterization. WI_GP is affected less by the altitude change.

RF11

2/12/2014

UHSAS was down for this flight with insufficient sample flow.

A discrepancy between WI, WIC and WI_GP appears to be caused by offset and unrealistic changes in CPITCH_GP.

SSRD and PSF / PSFC step change occurs at 07:28, which persists through the rest of the flight. The cause is not understood.

RF12

2/17/2014

UHSAS was down for this flight with insufficient sample flow.

The Omnistar service dropped out from 16:42 to the end of flight.

RF13

2/19/2014

UHSAS was down for this flight with insufficient sample flow.

RF14

2/22/2014

UHSAS was down for this flight with insufficient sample flow.

WI and WIC show a positive offset at the lower altitude leg from 10:15 to 10:55 due to an imperfect altitude dependent parameterization. WI_GP is affected less by the altitude change.

RF15

2/24/2014

RF15 was an inter-comparison flight with the DLR Falcon aircraft. Most of the flight was flown at altitudes lower than the one optimized for WI calculation, therefore an average non-zero WI offset is observed throughout the flight.

A loss of satellite reception just before takeoff resulted in a half-hour of sub-optimal OmniSTAR corrections while the differential system converged on the solution. The system did not establish full correction until 03:25.

UHSAS was down for this flight with insufficient sample flow.

RF16

2/28/2014

UHSAS was removed from the aircraft for repairs for this flight.

RF17

2/28/2014

UHSAS was removed from the aircraft for repairs for this flight.

_GP winds show one period of deviation from the radome winds. It is likely attributable to a CPITCH_GP drift that is not consistent and can’t be readily identified. This period is from 07:50 to 08:32.

The Omnistar service dropped out from 09:23 to 11:09. GGALT exhibit a step change at 10:53:55 when Omnistar recovered to data quality of 2. ALT_A and ALT_A2 should be used between 10:53:50 and 11:01.

RF18

2/28/2014

UHSAS was removed from the aircraft for repairs for this flight.

_GP winds show several periods of deviation from the radome winds. It is likely attributable to a CPITCH_GP drift that is not consistent. CPITCH_GP drift is apparent after every heading change on this flight.

RF19

2/28/2014

UHSAS was removed from the aircraft for repairs for this flight.

UI_GP and WD_GP are affected by heading measurement departures in the gust pod INS. Problematic periods with varying degree of THDG deviation are from 06:44 to 07:48.

RF20

2/28/2014

UHSAS pump was replaced prior to this flight, and probe tested during preflight.  Severe pegged flows during this flight make it difficult to assess performance, but there are no obvious flow dropouts.  There are some periods with very few particle counts, but these occur during times of pegged flow readings, so it isn't possible to confirm that sample flow was maintained in these periods.  Data from these "no particle" periods are therefore blanked.

WI and WIC in the altitude range close to 9,000 m are affected by the imperfect parameterization of the computation and exhibit a positive offset. This occurs during the time period from 05:12 to 05:25. WI and WIC and low altitudes demonstrate a negative offset.

The Omnistar service dropped out from 07:04 to 07:38.

VCSEL data gap exists from 03:34 to 03:41.

RF21

2/28/2014

UHSAS showed severe pegged flow readings during this flight.  Some periods with very few or no particle counts, but these occur during times of pegged flow readings, so it isn't possible to confirm that sample flow was maintained in these periods.  Data from these "no particle" periods are blanked, along with a few histogram spike events.

RF22

2/28/2014

UHSAS flow measurement much improved this flight following cable reseating by techs.  Sheath flow sagged ~20% at altitude, but remained at an acceptable level, and sample flow looked good the entire flight.  A few histogram spike events are blanked.

The Omnistar service dropped out from 08:42 – 08:50.

RF23

2/28/2014

UHSAS sheath flow sagged 15-20% at altitude, an acceptable decrease. Sample flow held throughout except 09:57:15-09:59:56 and 10:08:30-10:19:16, when it dropped to near zero, and each time recovered with a spike to a high value.  Sheath flow & UPRESS rose ~5% during these episodes.  It is not known if these are actual flow fluctuations or more of the connection problems.  Note that onboard tech (Cowan) reported 'in cloud' around the time of the first event, and during the second event that GV was climbing shortly after an icing incident. Some histogram spiking noted and blanked.

TAS_GP is not computed below 560 mbar and is missing from the middle part of this flight.

Altitude dependency of WIC and WI are also observed in mid and low altitude sections of this flight.

RF24

2/28/2014

UHSAS sheath flow sagged 15-20% at altitude, but sample flow was maintained throughout.  Still seeing occasional spikes in the histogram.

A discrepancy between WI, WIC and WI_GP appears to be caused by offset and unrealistic changes in CPITCH_GP.

RF25

2/28/2014

UHSAS sheath flow sagged 15-20% at altitude, and sample flow was maintained throughout except for a small drop at 06:02:25 - 06:03:08 (during climb-out) and a complete dropout at 12:04:45 - 12:07:30 (on descent). Histogram spikes noted and blanked.

A discrepancy between WI, WIC and WI_GP that is present throughout the flight appears to be caused by offset and unrealistic changes in CPITCH_GP.

All _GP winds are affected by the gust pod INS heading (CTHDG_GP) shift starting at 11:06 to the end of flight and shows a stair step change, which is not realistic and has been removed from the flight data.

RF26

2/28/2014

UHSAS sheath flow sagged ~20% at altitude, but sample flow held nicely throughout.  A few histogram spikes were noted and blanked.  A few instances of small-diameter noise also were blanked.

Technician notes

VARIABLE LIST

| ACINS | IRS Vertical Acceleration |
| --- | --- |
| ADIFR | Vertical Differential Pressure, Radome |
| ADIF_GP | Vertical Differential Pressure, Gust Pod |
| AKC_GP | Attack Angle, Diff. Pressure |
| AKRD | Attack Angle, Radome Diff. Pressure |
| AK_GP | Attack Angle, Diff. Pressure |
| ALT | IRS Altitude |
| ALT_A | Avionics GPS Altitude (MSL) |
| ATHR1 | Ambient Temperature, Deiced Right |
| ATHR2 | Ambient Temperature, Deiced Right |
| ATRL | Ambient Temperature, Radome Left |
| ATTACK | Attack Angle, Reference |
| ATX | Ambient Temperature, Reference |
| BDIFR | Horizontal Differential Pressure, Radome |
| BDIF_GP | Horizontal Differential Pressure, Gust Pod |
| BLATA | IRS Body Latitudal Acceleration |
| BLONGA | IRS Body Longitudal Acceleration |
| BNORMA | IRS Body Normal Acceleration |
| CNCNTS | TSI CN Counter Output |
| CNCONC | Condensation Nuclei (CN) Concentration |
| CNFLOW | Corrected BCN Counter Sample Flow Rate |
| CNPS | Pressure in butanol CN counter inlet |
| CONCU100_RWO | UHSAS Concentration, .1 micron and bigger |
| CONCU500_RWO | UHSAS Concentration, .5 micron and bigger |
| CONCU_RWO | UHSAS Concentration (all cells) |
| CONCV_VXL | VCSEL Moisture Number Density |
| CPITCH_GP | CMIGITS3 Aircraft Pitch |
| CPITCH_LAMS | CMIGITS3 Aircraft Pitch |
| CROLL_GP | CMIGITS3 Aircraft Roll |
| CROLL_LAMS | CMIGITS3 Aircraft Roll |
| CTHDG_GP | CMIGITS3 True Heading |
| CTHDG_LAMS | CMIGITS3 True Heading |
| CVEW_GP | CMIGITS3 Ground Speed Vector, East Component |
| CVEW_LAMS | CMIGITS3 Ground Speed Vector, East Component |
| CVNS_GP | CMIGITS3 Ground Speed Vector, North Component |
| CVNS_LAMS | CMIGITS3 Ground Speed Vector, North Component |
| CVSPD_GP | CMIGITS3 Verticle Speed |
| CVSPD_LAMS | CMIGITS3 Verticle Speed |
| DBARU_RWO | UHSAS Mean Particle Diameter |
| DISPU_RWO | UHSAS Dispersion (sigma/dbarx) |
| DPXC | Dew/Frost Point Temperature, Reference |
| DP_DPL | Dew/Frost Point Temperature |
| DP_DPR | Dew/Frost Point Temperature |
| DP_VXL | Dew/Frost Point Temperature |
| DVALUE | D-VALUE (GGALT - PALT) |
| EWX | Ambient Water Vapor Pressure, Reference |
| EW_DPL | Ambient Water Vapor Pressure |
| EW_DPR | Ambient Water Vapor Pressure |
| EW_VXL | Ambient Water Vapor Pressure (VCSEL) |
| GGALT | Reference GPS Altitude (MSL) |
| GGALTSD | Standard Deviation of Reference GPS Altitude (MSL) |
| GGLAT | Reference GPS Latitude |
| GGLATSD | Standard Deviation of Reference GPS Latitude |
| GGLON | Reference GPS Longitude |
| GGLONSD | Standard Deviation of Reference GPS Longitude |
| GGNSAT | Reference GPS number of satellites used in solution |
| GGQUAL | Reference GPS Qual, 0=inval,1=GPS,2=DGPS |
| GGSPD | Reference GPS Ground Speed |
| GGTRK | Reference GPS Track Angle |
| GGVEW | Reference GPS Ground Speed Vector, East Component |
| GGVNS | Reference GPS Ground Speed Vector, North Component |
| GGVSPD | Reference GPS Vertical Speed |
| GSF | IRS Aircraft Ground Speed |
| GSF_A | Avionics GPS Ground Speed |
| IWD | IRS Wind Direction |
| IWS | IRS Wind Speed |
| LAT | IRS Latitude |
| LATC | GPS-Corrected Inertial Latitude |
| LAT_A | Avionics GPS Latitude |
| LON | IRS Longitude |
| LONC | GPS-Corrected Inertial Longitude |
| LON_A | Avionics GPS Longitude |
| MACHF | Aircraft Mach Number, Fuselage |
| MACHR | Aircraft Mach Number, Radome |
| MACHX | Aircraft Mach Number, Reference |
| MACH_A | ADC Mach Number |
| MACH_GP | Aircraft Mach Number |
| MIRRTMP_CR2 | Cryogenic Hygrometer Mirror Temperature |
| MIRRTMP_DPL | Raw Dew/Frost Point Temperature |
| MIRRTMP_DPR | Raw Dew/Frost Point Temperature |
| MODE_GP | CMIGITS Mode - 2=init,4=align,7=nav |
| MODE_LAMS | CMIGITS Mode - 2=init,4=align,7=nav |
| MODE_VXL | VCSEL Mode |
| MR | Mixing Ratio, T-Electric |
| MR_CR2 | Cryogenic Hygrometer Mixing Ratio |
| NOSETMP | Radome Environmental Box Temperature |
| NPOSP_GP | CMIGITS Number of Position Measurements Processed |
| NPOSP_LAMS | CMIGITS Number of Position Measurements Processed |
| ONE | Constant value of 1. |
| PALT | NACA Pressure Altitude |
| PALTF | NACA Pressure Altitude |
| PALT_A | ADC Pressure Altitude |
| PCAB | Interior Cabin Static Pressure |
| PDUMPPR | Pressure of instrument exhaust duct, right |
| PITCH | IRS Aircraft Pitch Angle |
| PSF | Raw Static Pressure, Fuselage |
| PSFC | Corrected Static Pressure, Fuselage |
| PSX | Raw Static Pressure, Reference |
| PSXC | Corrected Static Pressure, Reference |
| PS_A | ADC Static Pressure |
| PS_GP | Raw Dynamic Pressure, Gust Pod |
| PVOLU_RWO | UHSAS Equivalent Volume |
| QCF | Raw Dynamic Pressure, Fuselage |
| QCFC | Corrected Dynamic Pressure, Fuselage |
| QCR | Raw Dynamic Pressure, Radome |
| QCRC | Corrected Dynamic Pressure, Radome |
| QCX | Raw Dynamic Pressure, Reference |
| QCXC | Corrected Dynamic Pressure, Reference |
| QC_A | ADC Impact Pressure |
| QC_GP | Raw Dynamic Pressure, Gust Pod |
| RHODT | Absolute Humidity, T-Electric |
| RHUM | Relative Humidity |
| RICE | Raw Icing-Rate Indicator |
| ROLL | IRS Aircraft Roll Angle |
| RTHR1 | Recovery Air Temperature, Deiced Right, HARCO |
| RTHR2 | Recovery Air Temperature, Deiced Right, HARCO |
| RTRL | Recovery Air Temperature, Radome Left |
| RTX | Recovery Temperature, Reference |
| RT_A | ADC Recovery Air Temperature |
| SOLAZ | Solar Azimuth Angle |
| SOLDE | Solar Declination Angle |
| SOLEL | Solar Elevation Angle |
| SOLZE | Solar Zenith Angle |
| SSLIP | Sideslip Angle, Reference |
| SSRD | Sideslip Angle, Radome Diff. Pressure |
| SS_GP | Sideslip Angle, Diff. Pressure |
| STATUS_CR2 | Cryogenic Hygrometer Status Flag (1 is good) |
| STATUS_VXL | VCSEL Status Code |
| TASF | Aircraft True Airspeed, Fuselage |
| TASHC | Aircraft True Airspeed, Humidity Corrected |
| TASR | Aircraft True Airspeed, Radome |
| TASX | Aircraft True Airspeed, Reference |
| TAS_A | ADC True Air Speed |
| TAS_GP | Aircraft True Airspeed |
| TCAB | Cabin Temperature at ADS Rack Location |
| TCNTU_RWO | UHSAS Total Counts (all cells) |
| THDG | IRS Aircraft True Heading Angle |
| THDG_IRS2 | IRS Aircraft True Heading Angle |
| THETA | Potential Temperature |
| THETAE | Equivalent Potential Temperature |
| THETAP | Pseudo-adiabatic Equivalent Potential Temperature |
| THETAV | Virtual Potential Temperature |
| TKAT | IRS Aircraft Track Angle |
| TKAT_A | Avionics GPS Track Angle |
| TVIR | Virtual Temperature |
| Time | time of measurement |
| UFLWC_RWO | UHSAS Corrected Flow |
| UI | Wind Vector, East Component |
| UIC | GPS-Corrected Wind Vector, East Component |
| UI_GP | Wind Vector, East Component |
| UX | Wind Vector, Longitudinal Component |
| UXC | GPS-Corrected Wind Vector, Longitudinal Component |
| UX_GP | Wind Vector, Longitudinal Component |
| VEW | IRS Ground Speed Vector, East Component |
| VEWC | GPS-Corrected Inertial Ground Speed Vector, East Component |
| VI | Wind Vector, North Component |
| VIC | GPS-Corrected Wind Vector, North Component |
| VI_GP | Wind Vector, North Component |
| VMR_VXL | Volume Mixing Ratio |
| VNS | IRS Ground Speed Vector, North Component |
| VNSC | GPS-Corrected Inertial Ground Speed Vector, North Component |
| VSPD | IRS Vertical Speed |
| VSPD_A | Avionics GPS Vertical Velocity |
| VY | Wind Vector, Lateral Component |
| VYC | GPS-Corrected Wind Vector, Lateral Component |
| VY_GP | Wind Vector, Lateral Component |
| WD | Horizontal Wind Direction |
| WDC | GPS-Corrected Horizontal Wind Direction |
| WD_GP | Horizontal Wind Direction |
| WI | Wind Vector, Vertical Gust Component |
| WIC | GPS-Corrected Wind Vector, Vertical Gust Component |
| WI_GP | Wind Vector, Vertical Gust Component |
| WS | Horizontal Wind Speed |
| WSC | GPS-Corrected Horizontal Wind Speed |
| WS_GP | Horizontal Wind Speed |
| ZERO | Constant value of 0. |