IDEAS-IV, GV 2013 project manager data QC report

Wing probes configuration

Wing probe configuration was changed from flight to flight during IDEAS-IV as outlined in the table below, in order to accommodate testing of more wing probes than can:

| Flight number | Left wing outboard (LWI) | Right wing outboard (RWO) |
| --- | --- | --- |
| RF01 | 25um 2DC ; CDP16 | 10um 2DC |
| RF02 | 25um 2DC ; CDP16 | 10um 2DC |
| RF03 | 25um 2DC ; CDP16 | 25um 2DC |
| RF04 | 25um 2DC ; CDP16 | 25um 2DC |
| RF05 | 25um 2DC ; CDP58 | 25um 2DC |
| RF06 | 25um 2DC ; CDP58 | 10um 2DC |
| RF07 | 25um 2DC ; SID2 | S100 |
| RF08 | 25um 2DC ; SID2 | S100 |
| RF09 | CDP58 ; SID2 | S100 |
| RF10 | CDP58 ; SID2 | CDP16 |

Data Quality Control

General:

- AKRD vs PITCH and, subsequently, WIC demonstrate altitude dependant offsets, as described in the ATTACK section above.

- Difference between QCF/QCR and QC_A can be as high as 6 mbar.

- Chilled mirror dewpointers exhibit occasional oscillations. DPR generally provided better data than DPL. DPL exhibits slower response and can lag DPR by seconds, especially at cold temperatures. Neither dewpointer is able to accurately measure frost point below -60C.

- ATRL had iced over on RF03 and was not flown on subsequent flights.

- Digital imagery had short dropouts on a number of flights, except as noted below.

Flight specific notes, IDEAS-IV GV 2013:

RF01: DP_VXL level shift at 20:36:15, presumably due to the line lock change. Level shift persisted through the rest of the flight. No digital imagery was recorded on this flight. Data from 10um 2D-C (*_RWIO) are bad for the entire flight.

RF02: Digital imagery recording started 2 hours after take off. 25um 2D-C (*_LWIO) experienced low detector voltage at some points, causing loss of data from 20:06:19 to 20:09:53. 10um 2D-C (*_RWIO) experienced detector saturation, causing loss of data from 19:56 to 21:10.

RF03: QCR was lost due to radome icing at 20:02 and did not recover for the rest of the flight. ADIFR (and all derived parameters) was lost due to radome icing at 21:34:33 and recovered at 22:37:42. ATRL was affected by icing starting at 20:46:08 and did not recover for the rest of the flight. VCSEL did not work most of the flight. TASX is affected by ADIFR icing and is not reliable in the current algorithm implementation until the dependency is changed; TAS_A is more accurate. VCSEL data are missing for most of this flight, and are bad where present. C1DC_LWIO, CONC1D_LWIO concentrations are elevated due to shattering on probe tips.  Rounded tips on this flight were installed for comparison to RWOI that was using anti-shatter tips.

RF04: VCSEL was not on the flight.

RF05: complete data loss from 17:39:50 to 17:43:30. DPL was not operating on this flight. VCSEL was not on the flight.

RF06: VCSEL was not on the flight.

RF07: Omnistar dropout from start of flight until 17:14. Radome icing was encountered at 18:49:43, which cleared at 20:04:07. QCR data were lost during this period. VCSEL was not on the flight.

RF08: No specific issues. VCSEL was not on the flight.

RF09: Radome icing was encountered at 21:46:12, blocking the QCR pressure port. QCR did not recover until the end of the flight. Brief Omnistar dropout was experienced at 19:54. VCSEL was not on the flight.

RF10: VCSEL was re-installed for this flight and operated nominally. No specific issues.

Technician Notes

Project - IDEAS-GV

Date 09-23-13

Flight Number - RF01

Block Out -1954

Takeoff - 2010

Prior to takeoff there were no AEROS displays. Chris traced it to the LICOR signal modifications he had made just prior to taxi.

DSM 313 has no IRIG-B or PPS

Cameras would not start, the program keeps core dumping.

No Piccarro data on any of the 4 onboard.

CR 2 was not plumbed up.

During flight the following was noticed:

PLWC and RICE were both  NAN.

We could not get a 2DC display on the L5 laptop or the L3 laptop.

CMIGITS stopped in flight at altitude. Chris rebooted DSM and it came back.

CDP sends house keeping but it does not look like it gives good histogram and concentrations.

At 1515 the ADS DSM303 had a serial card lock up.  All channels went red on the status page and stopped reporting data. A reboot fixed the problem. This is similar to issues we saw during MPEX with serial card lockups.

Power Readings GV

Time - 2033

Left Side A Bus

400 Hz A Phase  10.0 A  116V

400 Hz B Phase  8.1A  117V

400 Hz C Phase  14.1A  115 V

400 Hz AI A Phase  3.8 A  116V

400 Hz AI B Phase  0.0 A  117V

400 Hz AI C Phase  0.0 A  115V

60 Hz  19 A  115V

Right Side B Bus

400 Hz A Phase  12.1A  116V

400 Hz B Phase  14.9A  113V

400 Hz C Phase  10.5A  116V

400 Hz AI A Phase  0.0 A  116V

400 Hz AI B Phase  3.9A  113V

400 Hz AI C Phase  0.0 A  115V

60 Hz  13A 115V

28 VDC  14A   28.6V

RF 02

Tech: Kyle
9/26/13   

Takeoff:  19:59
Landing:  21:09

Not seeing PIC_H20.  Was informed we shouldn't and not to worry about it.
On the Status Page, cameras were green before takeoff.  I turned my seat around after takeoff and it was red but cameras still working.
ADIFR reading -15ish.  Doesn't seem right.  Probably a code issue.
It appears the HCR is inducing about a 70mV square wave pulse into RICE when the antenna is pointing down (180 degrees).  We tested at 60 and 90 degrees with no interference.
LWI and LWO DSM's PPS was red a lot.  A lot of sync toggles.

GV_IDEAS4_13

RF03

TECH:	Munnerlyn

DATE: 27 Sept 2013

19:05Z	 Wheels Up

19:15Z	 Restart Forward Camera

19:20Z	 Restart LWO DSM, note: air/water leak at VCSEL port has stopped. LWI DSM stopped and restarted on its own.

20:24Z	 Cycled power RWO DSM and VCSEL

20:58Z	 Power off RWO DSM for 5 min, then power on. Power cycle VCSEL.

21:18Z	 Turned off RWO 400Hz. Cycled power RWO DSM

21:45Z	 all angle of attack variables out. Note: LWI DSM intermittent out, shows no PPS

22:26Z	 HCR transmit off, Cycled power RWO DSM

22:50Z	 TTRL out, all other variables okay. Suspect ice

RF04

TECH:  D. Allbee

DATE:  10/1/2013

Taxi – 1125

Takeoff – 1130

Landed – 1533

PLWCC variable did not work. Was -32767

Forward Camera was not working for the majority of the flight. Did not turn on with W.O.W. Chris W. restarted it from the ground toward the end of the flight and it began working.

Cycled LWI DSM circuit breaker because PLWC and end diode voltages were not being displayed.

RF05

TECH:Irwin

DATE: 7 Oct 13

Flight took off just after 11 am local.  Landed about 3:15.

Flight headed up to North East where we proceeded to make racetracks (all left turns) for the majority of the flight.

At 17:40Z we lost all Aeros updates for 3 1/2 minutes, Kyle is now

telling me there is NO data recorded during that time!  One issue that

may have contributed to this (Webster) can comment, Mike S. had 2 2dc

imagines going on his single Aeros display with LAMS histograms going

and had issues with his display, once I saw what he was doing, I told

him to shut his down and restart with separate aero's displays for each

2dc and histogram.

2dc's showed large disparity in con's during flight, seems the new tips

are doing something well.

GV_IDEAS4_13

RF06

TECH: Munnerlyn

DATE: 8 Oct 2013

16:18Z	 Wheels Up

17:14Z	 radar moved from down to side

18:27Z	 HCR stopped transmitting, tophat noise goes away

18:35Z	 HCR starts transmitting

18:44Z RICE tophat noise

Project - IDEAS GV

Date 10/11/13

Flight Number - RF07

Takeoff at 1100 L 1700 Z

no issues on startup.

Block Out - 1701

Takeoff - 1705

1713 a single nopps on lwo

1835 Cloud penetrations.  Data on all probes.  Jeff reports SPP100 seems a little low.

1836 Saw a few no pps spikes. They were on dsm's 303, 304, 305 only. All other DSM's were quiet.

1844 to 1850 saw a few no pps on all dsm's at various times during the climb.

1853 Non modified timing cards no pps.

1915 Listened to 125 in cockpit. There was no interference.  I did notice a squeal when the radio was tuned to 133.6 but it went away when the plane turned and climbed. The pilots did not notice the squeal.  I am not sure what that was all about.

1923 Started speed run.

1939 QCR looking like alt not a QC. Might be icing in the line or on the radome? Looks like it iced up at 1856.

Forward camera iced up on decent.

2004 QCR recovered.

A few no PPS from all the un modified card on landing.

Land 2008

Block In 2010

Power Readings GV

Time - 1746

Left Side A Bus

400 Hz A Phase  10.8A  116V

400 Hz B Phase  7.8A  117V

400 Hz C Phase  12.6 A  115 V

400 Hz AI A Phase  3.8 A  116V

400 Hz AI B Phase  0.0 A  117V

400 Hz AI C Phase  0.0 A  115V

60 Hz  19 A  115V

Right Side B Bus

400 Hz A Phase  15.0A  116V

400 Hz B Phase  17.8A  113V

400 Hz C Phase  14.0A  116V

400 Hz AI A Phase  0.0 A  116V

400 Hz AI B Phase 	4.0A  113V

400 Hz AI C Phase  0.0 A  115V

60 Hz  27A  115V

28 VDC  6.0 A   28.8V

RF08

TECH:  D. Allbee

DATE:  10/16/2013

Taxi – 1252

Takeoff – 1257

Landed – 1623

Minor leak in ADIFR. Fixed during following maintenance day.

Issue with CMIGITS required several reboots of the DSM and cycling the power on the entire LWO pylon to bring it back up.

RF09

TECH: Ta

DATE: 10/17/13

Took off: 1:55PM

After take off the IRIG-B signal from DSM 304, 305 looked noisy, but after aircraft reached 39-40,000 feet all DSM IRIG signal looked good with straight lines.

CDP probe didn't produce any data at all from beginning, with ok from Jeff I reset LWI circuit breaker but it didn't work. Finally with Chris’ help from the ground. He reconfigured to the different port and we have CDP data coming back.

-PLWC didn't function right although It's never show dead straight line or -32767 value. However PLWC came back within 1.5 hrs later with 11.6 w value.

-Nobody remember turn on 3VCPI for this flight. Luckily I did remember 5 minutes before we moving to run way, therefore we still have 3VCPI data for this flight. Other than that all ADS instrument worked okay!

Landed: 5:14 pm

RF10

TECH: Irwin

DATE: Oct 21 2013

Take off delayed till 2 pm local.  Flight took off at 1:55 local, headed north.

No CMIGITS, no Picarro CO2, made announcement in cabin of these being off, no reply.

Asked Mike Spowart if he needed LAMS power restarted, he said no.

3VCPI according to Aaron B. had stuck bit on 2ds…killed program.  Restarting software did not fix, powering down computer did not fix, and powering down probe and computer seemed to fix the issue.

CDP’s seemed to track and measure same amounts amazingly well.  All else looked good.

Landed about 3:40 pm

Variable name list (typical for RF10; other flights vary slightly depending on wing store configuration)

| A2DTEMP_ADS0 | A2D Temperature |
| --- | --- |
| A2DTEMP_LWI | A2DTemperature |
| A2DTEMP_MPDB0 | A2DTemperature |
| A2DTEMP_RAD | A2D Temperature |
| A2DTEMP_RWO | A2DTemperature |
| ACGAIN_VXL | VCSEL AC Gain |
| ACINS | IRS Vertical Acceleration |
| ACINS_IRS2 | IRS Vertical Acceleration |
| ACINS_IRS3 | IRS Vertical Acceleration |
| ADIFR | Vertical Differential Pressure, Radome |
| ADIFRTEMP | Temperature of ADIFR Probe |
| AKRD | Attack Angle, Radome Diff. Pressure |
| ALT | IRS Altitude |
| ALT_A | Avionics GPS Altitude (MSL) |
| ALT_A2 | Avionics GPS Altitude (MSL) |
| ALT_IRS2 | IRS Altitude |
| ALT_IRS3 | IRS Altitude |
| AMPOW_VXL2 | VXL2 Ambient Laser Power |
| AMZERO_VXL2 | VXL2 Ambient DC Zero |
| AQRATIO | Al's Fudge Factor |
| ATHR1 | Ambient Temperature, Deiced Right |
| ATHR2 | Ambient Temperature, Deiced Right |
| ATITR_ITR | Ambient Temperature, In-cloud Temperature Radiometer |
| ATRL | Ambient Temperature, Radome Left |
| ATTACK | Attack Angle, Reference |
| ATX | Ambient Temperature, Reference |
| AT_A | ADC Ambient Air Temperature |
| AT_A2 | ADC Ambient Air Temperature |
| AT_VXL | VCSEL Ambient Temperature |
| AT_VXL2 | VXL2 Air Temperature |
| AVGTRNS_LWIO | CDP Average Transit Time |
| AVGTRNS_RWOI | CDP Average Transit Time |
| BALNC_DPL | Dew/Frost Point Balance |
| BALNC_DPR | Dew/Frost Point Balance |
| BALT1_A | ADC Baro Altitude |
| BALT1_A2 | ADC Baro Altitude |
| BALT2_A | ADC baro corrected alt #2 |
| BALT2_A2 | ADC baro corrected alt #2 |
| BAL_CR2 | Cryogenic Hygrometer Balance |
| BAROCOR1_A | ADC baro correction #1 mB |
| BAROCOR1_A2 | ADC baro correction #1 mB |
| BAROCOR2_A | ADC baro correction #2 mB |
| BAROCOR2_A2 | ADC baro correction #2 mB |
| BDIFR | Horizontal Differential Pressure, Radome |
| BDIFRTEMP | Temperature of BDIFR Probe |
| BENTEMP_ITR | ITR RTD 2 Counts |
| BKGM_RWI | 3VCPI Background Mean |
| BLATA | IRS Body Latitudal Acceleration |
| BLATA_IRS2 | IRS Body Latitudal Acceleration |
| BLATA_IRS3 | IRS Body Latitudal Acceleration |
| BLONGA | IRS Body Longitudal Acceleration |
| BLONGA_IRS2 | IRS Body Longitudal Acceleration |
| BLONGA_IRS3 | IRS Body Longitudal Acceleration |
| BNORMA | IRS Body Normal Acceleration |
| BNORMA_IRS2 | IRS Body Normal Acceleration |
| BNORMA_IRS3 | IRS Body Normal Acceleration |
| BRDT_CR2 | Cryogenic Hygrometer Board Temperature |
| CALT_LAMS | Reference CMIGITS3 Altitude (MSL) |
| CAS_A | ADC Computed Air Speed |
| CAS_A2 | ADC Computed Air Speed |
| CAVP_DPL | Dewpoint Cavity Pressure, Left |
| CAVP_DPR | Dewpoint Cavity Pressure, Right |
| CDP5VMON_LWIO | CDP 5 Vdc Monitor |
| CDP5VMON_RWOI | CDP 5 Vdc Monitor |
| CDPCBTMP_LWIO | CDP Control Board Temperature |
| CDPCBTMP_RWOI | CDP Control Board Temperature |
| CDPLBTMP_LWIO | CDP Laser Block Temperature |
| CDPLBTMP_RWOI | CDP Laser Block Temperature |
| CDPLSRC_LWIO | CDP Laser Current |
| CDPLSRC_RWOI | CDP Laser Current |
| CDPLSRP_LWIO | CDP Laser Power Monitor |
| CDPLSRP_RWOI | CDP Laser Power Monitor |
| CDPQBDW_LWIO | CDP Qualifier DT Bandwidth |
| CDPQBDW_RWOI | CDP Qualifier DT Bandwidth |
| CDPQBLINE_LWIO | CDP Qualifier Baseline |
| CDPQBLINE_RWOI | CDP Qualifier Baseline |
| CDPQTHRSH_LWIO | CDP Qualifier Dynamic Threshold |
| CDPQTHRSH_RWOI | CDP Qualifier Dynamic Threshold |
| CDPSBDW_LWIO | CDP Sizer DT Bandwidth |
| CDPSBDW_RWOI | CDP Sizer DT Bandwidth |
| CDPSBLINE_LWIO | CDP Sizer Baseline |
| CDPSBLINE_RWOI | CDP Sizer Baseline |
| CDPSTHRSH_LWIO | CDP Sizer Dynamic Threshold |
| CDPSTHRSH_RWOI | CDP Sizer Dynamic Threshold |
| CDPWBTMP_LWIO | CDP Wing Board Temperature |
| CDPWBTMP_RWOI | CDP Wing Board Temperature |
| CELLPRESS_L820 | Licor Cell Pressure |
| CELLTEMP_L820 | Licor Cell Temperature |
| CFT_CR2 | Cryogenic Hygrometer Coldfinger Temperature |
| CH4_PIC1301 | Picarro Methane Mixing Ratio |
| CLAT_LAMS | Reference CMIGITS3 Latitude |
| CLEAN_CR2 | Cryogenic Hygrometer Mirror Needs Cleaning Flag |
| CLON_LAMS | Reference CMIGITS3 Longitude |
| CNT1 | CN Counts |
| CNTRCUR_VXL | VCSEL Center Current |
| CO2_L820 | Licor Carbon Dioxide Mixing Ratio |
| CO2_PIC1301 | Picarro Carbon Dioxide Mixing Ratio |
| CONCD_LWIO | CDP Concentration (all cells) |
| CONCD_RWOI | CDP Concentration (all cells) |
| CONCU100_CVIU | UHSAS Concentration, 0.1 micron and bigger |
| CONCU500_CVIU | UHSAS Concentration, 0.5 micron and bigger |
| CONCU_CVIU | UHSAS Concentration (all cells) |
| CONCV_VXL | VCSEL Moisture Number Density |
| CONC_RWI | 3VCPI # Concentration |
| CPITCH_LAMS | Reference CMIGITS3 Aircraft Pitch |
| CROLL_LAMS | Reference CMIGITS3 Aircraft Roll |
| CTHDG_LAMS | Reference CMIGITS3 TRUE Heading |
| CVCFACT | CVI concentration factor |
| CVCWC | CVI cloud condensed water content |
| CVDP | CVI dew point |
| CVEW_LAMS | Reference CMIGITS3 Ground Speed Vector, East Component |
| CVF1 |  |
| CVF1WR | CVI supply flow control |
| CVFX0 |  |
| CVFX0WR | CVI excess flow control |
| CVFX1 |  |
| CVFX2 |  |
| CVFX2WR | CVI TDL flow control |
| CVFX3 |  |
| CVFX3WR | CVI impactor flow control |
| CVFX4 |  |
| CVFX4WR | CVI OPC flow control |
| CVFX5 | CVI User Flow |
| CVFX6 | CVI User Flow |
| CVFX7 | CVI User Flow |
| CVFX8 | CVI User Flow |
| CVFXFLOWS | CVI user flow index |
| CVINLET | CVI inlet flag, 0=CVI, 1=Total |
| CVNS_LAMS | Reference CMIGITS3 Ground Speed Vector, North Component |
| CVPCN | CVI Sample Pressure |
| CVRAD | CVI minimum cut radius |
| CVRH | CVI relative humidity |
| CVSPD_LAMS | Reference CMIGITS3 Verticle Speed |
| CVTAI |  |
| CVTCN | CVI Temperature by CNC |
| CVTDIFF | CVI time lag + 0.5 sec |
| CVTP | CVI Temperature at Probe |
| CVTS | CVI Temperature Sample Line |
| CVTT | CVI Temperature Tip |
| CVV0 | CVI valve 0 |
| CVV1 | CVI valve 1 |
| CVV2 | CVI valve 2 |
| CVV3 | CVI valve 3 |
| DAAMP_VXL2 | VXL2 Direct Abs Amplitude |
| DALOC_VXL2 | VXL2 Direct Abs Location |
| DBARD_LWIO | CDP Mean Particle Diameter |
| DBARD_RWOI | CDP Mean Particle Diameter |
| DBARU_CVIU | UHSAS Mean Particle Diameter |
| DBZD_LWIO | CDP Calculated Reflectivity |
| DBZD_RWOI | CDP Calculated Reflectivity |
| DELTAT_LWIO | CDP Time between last sample and this sample |
| DELTAT_RWOI | CDP Time between last sample and this sample |
| DETTEMP_ITR | ITR Detector Temperature |
| DETV1_ITR | ITR Detector Voltage 1 |
| DETV2_ITR | ITR Detector Voltage 2 |
| DISPD_LWIO | CDP Dispersion (sigma/dbarx) |
| DISPD_RWOI | CDP Dispersion (sigma/dbarx) |
| DISPU_CVIU | UHSAS Dispersion (sigma/dbarx) |
| DPXC | Dew/Frost Point Temperature, Reference |
| DP_DPL | Dew/Frost Point Temperature |
| DP_DPR | Dew/Frost Point Temperature |
| DP_VXL | Dew/Frost Point Temperature |
| DP_VXL2 | VXL2 Dew Point |
| DRFTA | IRS Drift Angle |
| DRFTA_IRS2 | IRS Drift Angle |
| DRFTA_IRS3 | IRS Drift Angle |
| DVALUE | D-VALUE (GGALT - PALT) |
| DVALUE_GMN | D-VALUE (GGALT - PALT) |
| EWX | Ambient Water Vapor Pressure, Reference |
| EW_DPL | Ambient Water Vapor Pressure |
| EW_DPR | Ambient Water Vapor Pressure |
| EW_VXL | Ambient Water Vapor Pressure (VCSEL) |
| EXHPERR_LAMS | Expected Horizontal Position Error |
| EXVELERR_LAMS | Expected Velocity Error |
| EXVPERR_LAMS | Expected Vertical Position Error |
| F2DIO1_LWIO | Fast 2DC End Diode Voltage #1 |
| F2DIO1_RWOI | Fast 2DC End Diode Voltage #1 |
| F2DIO32_LWIO | Fast 2DC End Diode Voltage #32 |
| F2DIO32_RWOI | Fast 2DC End Diode Voltage #32 |
| F2DIO64_LWIO | Fast 2DC End Diode Voltage #64 |
| F2DIO64_RWOI | Fast 2DC End Diode Voltage #64 |
| FOMINF_LAMS | Figure of Merit Information - see manual |
| FRMS_RWI | 3VCPI Frames per Second |
| GGALT | Reference GPS Altitude (MSL) |
| GGALTSD | Standard Deviation of Reference GPS Altitude (MSL) |
| GGALT_GMN | Reference GPS Altitude (MSL) |
| GGDAGE | Time in seconds since last DGPS update |
| GGDAGE_GMN | Time in seconds since last DGPS update |
| GGEOIDHT | Reference GPS Height of geoid (MSL) above WGS84 ellipsoid |
| GGEOIDHT_GMN | Reference GPS Height of geoid (MSL) above WGS84 ellipsoid |
| GGHORDIL | Reference GPS Horizontal dilution of position |
| GGHORDIL_GMN | Reference GPS Horizontal dilution of position |
| GGLAT | Reference GPS Latitude |
| GGLATSD | Standard Deviation of Reference GPS Latitude |
| GGLAT_GMN | Reference GPS Latitude |
| GGLON | Reference GPS Longitude |
| GGLONSD | Standard Deviation of Reference GPS Longitude |
| GGLON_GMN | Reference GPS Longitude |
| GGNSAT | Reference GPS number of satellites used in solution |
| GGNSAT_GMN | Reference GPS number of satellites used |
| GGQUAL | Reference GPS Qual, 0=inval,1=GPS,2=DGPS,5=OMNISTAR |
| GGQUAL_GMN | Reference GPS Qual, 0=inval,1=GPS,2=DGPS |
| GGREFID | DGPS station ID number |
| GGREFID_GMN | DGPS station ID number |
| GGRepLag | GPS reporting lag: (data sys time tag of NMEA receipt)-(NMEA time) |
| GGRepLag_GMN | GPS reporting lag: (data sys time tag of NMEA receipt)-(NMEA time) |
| GGSECSDAY_GMN | Reference GPS seconds of day |
| GGSPD | Reference GPS Ground Speed |
| GGSPD_GMN | Reference GPS Ground Speed |
| GGSTATUS | Reference GPS rcvr status: 1=OK(A), 0=warning(V) |
| GGSTATUS_GMN | Reference GPS rcvr status: 1=OK(A), 0=warning(V) |
| GGTRK | Reference GPS Track Angle |
| GGTRK_GMN | Reference GPS Track Angle |
| GGVEW | Reference GPS Ground Speed Vector, East Component |
| GGVEW_GMN | Reference GPS Ground Speed Vector, East Component |
| GGVNS | Reference GPS Ground Speed Vector, North Component |
| GGVNS_GMN | Reference GPS Ground Speed Vector, North Component |
| GGVSPD | Reference GPS Vertical Speed |
| GSF | IRS Aircraft Ground Speed |
| GSF_A | Avionics GPS Ground Speed |
| GSF_A2 | Avionics GPS Ground Speed |
| GSF_IRS2 | IRS Aircraft Ground Speed |
| GSF_IRS3 | IRS Aircraft Ground Speed |
| GSTAT_A | Avionics GPS Sensor Status |
| GSTAT_A2 | Avionics GPS Sensor Status |
| GTIME_A | Avionics GPS UTC Measure Time |
| GTIME_A2 | Avionics GPS UTC Measure Time |
| H0_RWI | 3VCPI HElement0 |
| H127_RWI | 3VCPI HElement127 |
| H2OR | TDL Water Concentration |
| H2O_CVIP | Picarro Water Mixing Ratio |
| H2O_PICCU | Picarro Water Mixing Ratio |
| H64_RWI | 3VCPI HElement64 |
| HK12V_VXL2 | VXL2 12V |
| HK15MV_VXL2 | VXL2 15MV |
| HK15PV_VXL2 | VXL2 15PV |
| HK5V_VXL2 | VXL2 5V |
| HKREC_RWI | HK Received |
| HMSK_RWI | H Mask |
| HPRT_RWI | 3VCPI Horizontal Particles Detected |
| IRIG_Status_303 | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_304 | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_305 | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_313 | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_LWI | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_LWO | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Status_RWO | IRIG status: bit 0=SYNC,1=NOCODE,2=NOPPS,3=NOMAJT,4=NOYEAR,5=NOSYNC |
| IRIG_Tdiff_303 | IRIG-UNIX clock diff |
| IRIG_Tdiff_304 | IRIG-UNIX clock diff |
| IRIG_Tdiff_305 | IRIG-UNIX clock diff |
| IRIG_Tdiff_313 | IRIG-UNIX clock diff |
| IRIG_Tdiff_LWI | IRIG-UNIX clock diff |
| IRIG_Tdiff_LWO | IRIG-UNIX clock diff |
| IRIG_Tdiff_RWO | IRIG-UNIX clock diff |
| IWD | IRS Wind Direction |
| IWD_IRS2 | IRS Wind Direction |
| IWD_IRS3 | IRS Wind Direction |
| IWS | IRS Wind Speed |
| IWS_IRS2 | IRS Wind Speed |
| IWS_IRS3 | IRS Wind Speed |
| LASERSET_VXL2 | VXL2 Laser Setpoint |
| LASERT_VXL2 | VXL2 Laser Temperature |
| LAT | IRS Latitude |
| LATC | GPS-Corrected Inertial Latitude |
| LATF_A | Avionics GPS Latitude Fine |
| LATF_A2 | Avionics GPS Latitude Fine |
| LAT_A | Avionics GPS Latitude |
| LAT_A2 | Avionics GPS Latitude |
| LAT_IRS2 | IRS Latitude |
| LAT_IRS3 | IRS Latitude |
| LCTV | TDL line center Trans value |
| LON | IRS Longitude |
| LONC | GPS-Corrected Inertial Longitude |
| LONF_A | Avionics GPS Longitude Fine |
| LONF_A2 | Avionics GPS Longitude Fine |
| LON_A | Avionics GPS Longitude |
| LON_A2 | Avionics GPS Longitude |
| LON_IRS2 | IRS Longitude |
| LON_IRS3 | IRS Longitude |
| LSRINT_VXL | VCSEL Averaged Laser Intensity |
| LSRTMP_VXL | VCSEL Laser Temperature |
| MACHF | Aircraft Mach Number, Fuselage |
| MACHR | Aircraft Mach Number, Radome |
| MACHX | Aircraft Mach Number, Reference |
| MACH_A | ADC Mach Number |
| MACH_A2 | ADC Mach Number |
| MHDG | IRS Aircraft Magnetic Heading Angle |
| MHDG_IRS2 | IRS Aircraft Magnetic Heading Angle |
| MHDG_IRS3 | IRS Aircraft Magnetic Heading Angle |
| MIRRORT_CR2 | Cryogenic Hygrometer Mirror Temperature |
| MIRRTMP_DPL | Raw Dew/Frost Point Temperature |
| MIRRTMP_DPR | Raw Dew/Frost Point Temperature |
| MODE_LAMS | CMIGITS Mode - 2=init,4=align,7=nav |
| MODE_VXL | VCSEL Mode |
| MODE_VXL2 | VXL2 Mode |
| MOTRSPD_ITR | ITR Motor Speed |
| MR | Mixing Ratio, T-Electric |
| MR2H_CVIP | Picarro Deuterium Mixing Ratio |
| MR2H_PICCU | Picarro Deuterium Mixing Ratio |
| MR_CR2 | Cryogenic Hygrometer Mixing Ratio |
| NEG12_RWI | 3VCPI -12 Volts |
| NEG7_RWI | -7V |
| NOSETMP | Radome Environmental Box Temperature |
| NPOSP_LAMS | Number of Position Measurements Processed |
| NSAT_LAMS | Number of Satellites Tracked |
| NVELP_LAMS | Number of Velocity Measurements Processed |
| O18_CVIP | Picarro O18 Mixing Ratio |
| O18_PICCU | Picarro O18 Mixing Ratio |
| ONE | Constant value of 1 |
| OVFLW_LWIO | CDP AtoD Converter Overflow/Overrange |
| OVFLW_RWOI | CDP AtoD Converter Overflow/Overrange |
| PALT | NACA Pressure Altitude |
| PALTF | NACA Pressure Altitude |
| PALT_A | ADC Pressure Altitude |
| PALT_A2 | ADC Pressure Altitude |
| PCAB | Interior Cabin Static Pressure |
| PDLR | TDL Sample cell pressure |
| PDRY | Power Dry Calculated |
| PDUMPPL | Pressure of instrument exhaust duct, left |
| PDUMPPR | Pressure of instrument exhaust duct, right |
| PINT_RWI | Internal Pressure |
| PITCH | IRS Aircraft Pitch Angle |
| PITCH_IRS2 | IRS Aircraft Pitch Angle |
| PITCH_IRS3 | IRS Aircraft Pitch Angle |
| PKPOS_VXL | VCSEL Peak Position |
| PLWC | Raw PMS-King Liquid Water Content Output |
| PLWCC | Corrected PMS-King Liquid Water Content |
| PLWCD_LWIO | CDP Water/Ice Content |
| PLWCD_RWOI | CDP Water/Ice Content |
| POS5_RWI | 3VCPI 5 Volts |
| POS7_RWI | +7V |
| PPS_303 | 1-PPS Signal |
| PPS_LWI | 1-PPS Signal |
| PRCTVAL_RWI | 3VCPI % Valid |
| PREGAIN_VXL | VCSEL Pre Gain |
| PRESS_VXL2 | VXL2 Pressure |
| PSF | Raw Static Pressure, Fuselage |
| PSFC | Corrected Static Pressure, Fuselage |
| PSX | Raw Static Pressure, Reference |
| PSXC | Corrected Static Pressure, Reference |
| PS_A | ADC Static Pressure |
| PS_A2 | ADC Static Pressure |
| PS_VXL | VCSEL Ambient Pressure |
| PTHGT_VXL2 | VXL2 PT Height |
| PT_A | ADC Total Pressure |
| PT_A2 | ADC Total Pressure |
| PVOLU_CVIU | UHSAS Equivalent Volume |
| PWM_CR2 | Cryogenic Hygrometer PWM |
| P_CR2 | Cryogenic Hygrometer Pressure |
| Q1_VXL2 | VXL2 Size of Queue 1 |
| Q2_VXL2 | VXL2 Size of Queue 2 |
| Q3_VXL2 | VXL2 Size of Queue 3 |
| QCF | Raw Dynamic Pressure, Fuselage |
| QCFC | Corrected Dynamic Pressure, Fuselage |
| QCFTEMP | Temperature of QCF Probe |
| QCR | Raw Dynamic Pressure, Radome |
| QCRC | Corrected Dynamic Pressure, Radome |
| QCRTEMP | Temperature of QCR Probe |
| QCX | Raw Dynamic Pressure, Reference |
| QCXC | Corrected Dynamic Pressure, Reference |
| QC_A | ADC Impact Pressure |
| QC_A2 | ADC Impact Pressure |
| RAWCONC_VXL | VCSEL Moisture Number Density |
| REFFD_LWIO | CDP Effective Radius |
| REFFD_RWOI | CDP Effective Radius |
| REJDOF_LWIO | CDP Rejected Particle Count (Outside Depth-of-Field) |
| REJDOF_RWOI | CDP Rejected Particle Count (Outside Depth-of-Field) |
| REJDOF_SIDS | SIDS Rejected Particle Count |
| RHODT | Absolute Humidity, T-Electric |
| RHUM | Relative Humidity |
| RICE | Raw Icing-Rate Indicator |
| ROLL | IRS Aircraft Roll Angle |
| ROLL_IRS2 | IRS Aircraft Roll Angle |
| ROLL_IRS3 | IRS Aircraft Roll Angle |
| RPA_VXL2 | VXL2 Ref Peak Amplitude |
| RPL_VXL2 | VXL2 Ref Peak Location |
| RSTB | Radiometric Surface Temperature |
| RTHR1 | Recovery Air Temperature, Deiced Right, HARCO |
| RTHR2 | Recovery Air Temperature, Deiced Right, HARCO |
| RTRL | Recovery Air Temperature, Radome Left |
| RTX | Recovery Air Temperature, Reference |
| RT_A | ADC Recovery Air Temperature |
| RT_A2 | ADC Recovery Air Temperature |
| SEQ1_LAMS | LAMS Sequence Number, Beam 1 |
| SEQ2_LAMS | LAMS Sequence Number, Beam 1 |
| SEQ3_LAMS | LAMS Sequence Number, Beam 1 |
| SETTMP_VXL | VCSEL Set Temperature |
| SIZM50_RWI | 3VCPI Mean Particle Size for all > 50 microns |
| SIZMAX_RWI | 3VCPI Max Particle Size |
| SIZM_RWI | 3VCPI Mean Particle Size |
| SMPLSRINT_VXL | VCSEL Sample Laser Intensity |
| SNKTEMP_ITR | ITR RTD 3 Counts |
| SOLAZ | Solar Azimuth Angle |
| SOLDE | Solar Declination Angle |
| SOLEL | Solar Elevation Angle |
| SOLPOS | Solenoid state |
| SOLZE | Solar Zenith Angle |
| SSLIP | Sideslip Angle, Reference |
| SSRD | Sideslip Angle, Radome Diff. Pressure |
| STATUS_CR2 | Cryogenic Hygrometer Status Flag (1 is good) |
| STATUS_VXL | VCSEL Status Code |
| STATVALID_LAMS | Status Validity - BIT - see manual |
| STRTEMP_ITR | ITR RTD 1 Counts |
| SVOL_RWI | 3VCPI Sample Volume |
| TAMP_RWI | 3VCPI Amp Temperature |
| TASF | Aircraft TRUE Airspeed, Fuselage |
| TASHC | Aircraft TRUE Airspeed, Humidity Corrected |
| TASR | Aircraft TRUE Airspeed, Radome |
| TASX | Aircraft TRUE Airspeed, Reference |
| TAS_A | ADC TRUE Air Speed |
| TAS_A2 | ADC TRUE Air Speed |
| TBLK_RWI | 3VCPI Optics Block Temperature |
| TBRD_RWI | DSP Board Temp |
| TCAB | Cabin Temperature at ADS Rack Location |
| TCC_VXL | VCSEL Thermoelectric Cooler Current |
| TCNTD_LWIO | CDP Total Counts (all cells) |
| TCNTD_RWOI | CDP Total Counts (all cells) |
| TCNTU_CVIU | UHSAS Total Counts (all cells) |
| TDLlaser | TDL Raw laser pwr at line center |
| TDLline | TDL H2O line position in spectrum |
| TDLsignal | TDL Peak-to-peak raw 2f amplitude |
| TDLzero | TDL zero level |
| TDSP_RWI | 3VCPI DSP Temperature |
| TEC_DPL | Dew/Frost Point TEC |
| TEC_DPR | Dew/Frost Point TEC |
| THDG | IRS Aircraft TRUE Heading Angle |
| THDG_IRS2 | IRS Aircraft TRUE Heading Angle |
| THDG_IRS3 | IRS Aircraft TRUE Heading Angle |
| THETA | Potential Temperature |
| THETAE | Equivalent Potential Temperature |
| THETAP | Pseudo-adiabatic Equivalent Potential Temperature |
| THETAQ | Wet Equivalent Potential Temperature |
| THETAV | Virtual Potential Temperature |
| THLAS_RWI | H Laser Temp |
| THRXARM_RWI | H RX Arm Temp |
| THTSK | TDL TEC heatsink temperature |
| THTXARM_RWI | H TX Arm Temp |
| TKAT | IRS Aircraft Track Angle |
| TKAT_A | Avionics GPS Track Angle |
| TKAT_A2 | Avionics GPS Track Angle |
| TKAT_IRS2 | IRS Aircraft Track Angle |
| TKAT_IRS3 | IRS Aircraft Track Angle |
| TNOSE_RWI | 3VCPI Nose Temperature |
| TPS_RWI | Power Supply Temp |
| TRDIFF_VXL2 | VXL2 Trough Difference |
| TRSTB | RSTB Sensor Head Temperature |
| TRWID_VXL2 | VXL2 Trough Width |
| TTDLR | TDL Sample cell Temperature |
| TTENC | TDL Enclosure Temperature |
| TVIR | Virtual Temperature |
| TVLAS_RWI | V Laser Temp |
| TVRXARM_RWI | V RX Arm Temp |
| TVTXARM_RWI | V TX Arm Temp |
| Time | time of measurement |
| UBTMP_CVIU | UHSAS Temperature in Canister |
| UCURR_CVIU | UHSAS Laser Current |
| UDIFF_CVIU | UHSAS Absolute Pressure in Canister, signal |
| UFLWC_CVIU | UHSAS Corrected Flow |
| UHSAS_CNTS_CVIU | Total Counts |
| UI | Wind Vector, East Component |
| UIC | GPS-Corrected Wind Vector, East Component |
| UPRESS_CVIU | UHSAS Absolute Pressure in Canister |
| UREF_CVIU | UHSAS Reference Intensity |
| USCAT_CVIU | UHSAS Background Scatter |
| USHFLW_CVIU | UHSAS Air Sheath Flow |
| USMPFLW_CVIU | UHSAS Sample Flow |
| UTEMP_CVIU | UHSAS Temperature in Canister, signal |
| UX | Wind Vector, Longitudinal Component |
| UXC | GPS-Corrected Wind Vector, Longitudinal Component |
| V0_RWI | 3VCPI VElement0 |
| V127_RWI | 3VCPI VElement127 |
| V64_RWI | 3VCPI VElement64 |
| VALCOHIGH | Valco HIGH valve position |
| VALCOLOW | Valco LOW valve position |
| VEW | IRS Ground Speed Vector, East Component |
| VEWC | GPS-Corrected Inertial Ground Speed Vector, East Component |
| VEW_A | Avionics GPS Ground Speed Vector, East Component |
| VEW_A2 | Avionics GPS Ground Speed Vector, East Component |
| VEW_IRS2 | IRS Ground Speed Vector, East Component |
| VEW_IRS3 | IRS Ground Speed Vector, East Component |
| VI | Wind Vector, North Component |
| VIC | GPS-Corrected Wind Vector, North Component |
| VMR_VXL | Volume Mixing Ratio |
| VMSK_RWI | V Mask |
| VNS | IRS Ground Speed Vector, North Component |
| VNSC | GPS-Corrected Inertial Ground Speed Vector, North Component |
| VNS_A | Avionics GPS Ground Speed Vector, North Component |
| VNS_A2 | Avionics GPS Ground Speed Vector, North Component |
| VNS_IRS2 | IRS Ground Speed Vector, North Component |
| VNS_IRS3 | IRS Ground Speed Vector, North Component |
| VPRT_RWI | 3VCPI Vertical Particles Detected |
| VSPD | IRS Vertical Speed |
| VSPD_A | Avionics GPS Vertical Velocity |
| VSPD_A2 | Avionics GPS Vertical Velocity |
| VSPD_IRS2 | IRS Vertical Speed |
| VSPD_IRS3 | IRS Vertical Speed |
| VY | Wind Vector, Lateral Component |
| VYC | GPS-Corrected Wind Vector, Lateral Component |
| WD | Horizontal Wind Direction |
| WDC | GPS-Corrected Horizontal Wind Direction |
| WI | Wind Vector, Vertical Gust Component |
| WIC | GPS-Corrected Wind Vector, Vertical Gust Component |
| WOW_A | ADC Weight On Wheels |
| WOW_A2 | ADC Weight On Wheels |
| WS | Horizontal Wind Speed |
| WSC | GPS-Corrected Horizontal Wind Speed |
| WT_VXL2 | VXL2 Wall Temperature |
| ZERO | Constant value of 0 |