RF05

TECH: Cowan

Date 08/20/09

Pre-flight:  Nimbus died (display disappeard). Also LWO DSM stopped. Got Nimbus back but could not get LWO DSM to boot.

Dew Point sensors were still flooded from prior flight. Waited until after takeoff to turn on.  Good data after 0115

0119 – RWM DSM Stopped, Power cycled.

0125 – RWM DSM Stopped again

0129 – RWM back up on it’s own.

0232 – RWO DSM A/D temp reporting -32767, DSM is still connected.

0233 – Dew Point displays updated until about 0230 then both stopped.

0405 - LWI DSM found to have No Year Error, Power cycled.

0449 – Nimbus died, window closed, restarted. Satcom lost comms when I used page Chris button.

0600 – Nimbus died again, restarted DSM server and Nimbus per Chris’s request.

0654 – Found that restarting DSM server fixed strange -32767 errors.

During this flight at 0230 something happened.  After that time there were various sensors or groups of sensors that would report occasional or constant -32767. The data file was also corrupted at that point.  Restarting DSM server fixed the errors and I thinkt e file after that point.  I would suggest that in the future others reset DSM server when there are sensors intermittently reporting -32767.

Power readings:

Left Side A Bus

400 Hz

Phase A 6.1A 115V

Phase B 5.9A 116V
Phase C 6.8A 115V

400 Hz AI

Phase A 0A 115V

Phase B 0A 116V
Phase C 0A 115V

60Hz 117V 2A

Right Side B Bus

400 Hz

Phase A 6.1A 115V

Phase B 9.8A 113V
Phase C 6.7A 115V

400 Hz AI

Phase A 0A 115V

Phase B 4A 113V
Phase C 0A 115V

60Hz 115V 9A

28V

28.9V  19A