RF08

TECH: Cowan

1/23/09 CHC to Tahiti

VCSEL stuck at DP 100, should be checked prior to Taxi.

2102 Take off

2106 VCSEL stuck at 100% suspect something on the mirror.

2204 not seeing any laser intensity, suspect either something on the mirror or a clogged hole.  Hoping that something will wash it off. Running the instrument to keep it warm.

2318 Lost Satcom, did not come back.  Seeing an HSU fail light. Reset cockpit switch to get it to restart.

0041 Satcom drop out, had All lights out, then Service, MPDS and ISDN all came back, and it redailed. This is what should happen not the HSU failure light.

0113 Power readings

Left Side A Bus

400 Hz Phase A 13.4 A (1A Jump) 115 V

400 Hz Phase B 11.7 (1A Jump) 117 V

400 Hz Phase C 15.0 (1A Jump) 115 V

400 Hz AI Phase A 0A

400 Hz AI Phase B 0A

400 Hz AI Phase C 0A

60 Hz 116V 21A

Right Side B Bus

400 Hz Phase A 8.5 (1A Jump) 114V

400 Hz Phase B 10.0 (1A Jump) 114V

400 Hz Phase C 8.2 (1A Jump) 115V

400 Hz AI Phase A 0A

400 Hz AI Phase B 3.9A 113V

400 Hz AI Phase C 0A

28V AI 29V 13A

0152 DPL has rebalance light lit but seems to be tracking.  Leaving it alone for now.

0227 Good rain shower seeing water on the fuselage

0231 DPL Service Mirror light on, resetting

0301 DPL Service Mirror light on resetting

0330 0245 Major Icing of Radome. Lost ADIFR, BDIFR and QCR.  From 0245 to 0305 most stuff is bad, from 0305 to 0330 QCR still bad. Also at 0245 there were big temp spikes.  I suspect that we iced up the temp probes especially TTRF.

0401 QCR responding.

0414 Landing