TF03

TECH: Cowan

1/6/08

1640 Take off

1642 The Dew Point sensors seem to not be reacting the same on take off.

1648 Dew Point Right is controlling and Dew Point Left is not controlling, There is a 10 C difference between them.

1655 Looks like the 2DC has a stuck bit.

1728 Not seeing MTP-PC on the DSM server page

1739 started Google Earth and it did not connect to the ground.  This is a good thing.

1758 On descent Left Dew point Oscillated but the right was much slower to respond.

Post flight – Found an issue with the Right Dew Point flag.  Small piece of blocking material had moved and covered the exit hole blocking off the flow.  New flag installed for the project.