TF04

TECH: Cowan

DATE 09/14/09

Pre-flight

TTFR was removed for calibration. No data on the flight.

ADIF-GP was replaced before the flight. Read NAN preflight. Suspect transducer not pre-programmed.

All DSM’s had NOCODE error.  That means no IRIG-B.  This was fixed during flight.

No Camera’s during the flight.  There is a Kernal/Firewire driver mismatch.

2235 Cycled MTP Operate switch by mistake.

2243 Found RICE to be NAN.

2244 LWO DSM stopped.  Reset breaker to bring it back.

2301 NOCODE to DSM’s fixed by pushing in on IRIG Amp. Cord was loose.

2302 RICE now working.  Found it strange that RICE did not work but TT’s did work with NOCODE Error.

2308 LWO DSM stopped. Reset breaker to bring it back.

0008 LWO DSM stopped. Unable to reset due to turbulence.

0014 LWO DSM working again.  I did nothing to it, it came back on it’s own.

0035 Landing

I find it interesting that the Viper A/D cards were not effected by the NOCODE error yet the Vulcan A/D cards were not working.